Code used in Project

#define BLYNK_TEMPLATE_ID   "TMPL3pysIwKEL"

#define BLYNK_TEMPLATE_NAME "Robot"

#define BLYNK_AUTH_TOKEN    "z4YrYq5wt16OdNgyB2wK6J2cXfux5aRK"

#include <WiFi.h>

#include <BlynkSimpleEsp32.h>

#include <Wire.h>

#include <Adafruit_PWMServoDriver.h>

char ssid[] = "Bandham437";

char pass[] = "logout437";

// --- Pin Definitions ---

#define ENA 13

#define IN1 12

#define IN2 14

#define ENB 25

#define IN3 27

#define IN4 26

#define TRIG 5

#define ECHO 18

#define MQ2_PIN 34

// --- Rescue & Fire Pins ---

#define METAL_PIN 32   // Digital Input (PNP: HIGH = Metal)

#define FIRE_PIN 33    // Digital Input (Active LOW)

#define RELAY_PIN 15   // Digital Output (Active LOW)

Adafruit_PWMServoDriver pwm = Adafruit_PWMServoDriver();

BlynkTimer timer;

#define SERVOMIN  150

#define SERVOMAX  600

int joyX = 0, joyY = 0;

bool isRecoiling = false;

int gasThreshold = 1200;

// --- Alert Locks ---

bool gasAlertSent = false;

bool metalAlertSent = false;

bool fireAlertSent = false;

bool manualPumpOverride = false; // V10

void setup() {

Serial.begin(115200);

pinMode(IN1, OUTPUT); pinMode(IN2, OUTPUT);

pinMode(IN3, OUTPUT); pinMode(IN4, OUTPUT);

pinMode(TRIG, OUTPUT); pinMode(ECHO, INPUT);

ledcAttach(ENA, 20000, 8);

ledcAttach(ENB, 20000, 8);

pinMode(METAL_PIN, INPUT);

pinMode(FIRE_PIN, INPUT);

pinMode(RELAY_PIN, OUTPUT);

digitalWrite(RELAY_PIN, HIGH); // Pump OFF at boot

pwm.begin();

pwm.setPWMFreq(60);

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

timer.setInterval(100L, checkDistance);

timer.setInterval(500L, checkRescueSensors);

timer.setInterval(1000L, checkGasSensor);

}

void checkGasSensor() {

int gasValue = analogRead(MQ2_PIN);

Blynk.virtualWrite(V6, gasValue);

if (gasValue > gasThreshold && !gasAlertSent) {

Blynk.logEvent("gas_alert", "EMERGENCY: Gas/Smoke Detected!");

gasAlertSent = true;

}

if (gasValue < (gasThreshold - 200)) {

gasAlertSent = false;

}

}

void checkRescueSensors() {

// PNP Sensor Logic: HIGH means metal is detected

bool metalDetected = (digitalRead(METAL_PIN) == HIGH);

Blynk.virtualWrite(V8, metalDetected ? 1 : 0);

if (metalDetected && !metalAlertSent) {

Blynk.logEvent("metal_detected", "ALERT: Metal Object Located!");

metalAlertSent = true;

} else if (!metalDetected) {

metalAlertSent = false;

}

// Fire Detection Logic: LOW means fire is detected

bool fireDetected = (digitalRead(FIRE_PIN) == LOW);

Blynk.virtualWrite(V9, fireDetected ? 1 : 0);

// Pump triggers on fire OR manual override

if (fireDetected || manualPumpOverride) {

digitalWrite(RELAY_PIN, LOW);

if (fireDetected && !fireAlertSent) {

Blynk.logEvent("fire_alert", "FIRE DETECTED! WATER PUMP ENGAGED!");

fireAlertSent = true;

}

} else {

digitalWrite(RELAY_PIN, HIGH);

fireAlertSent = false;

}

}

BLYNK_WRITE(V10) {

manualPumpOverride = param.asInt();

}

BLYNK_WRITE(V2) { pwm.setPWM(0, 0, map(param.asInt(), 0, 180, SERVOMIN, SERVOMAX)); }

BLYNK_WRITE(V3) { pwm.setPWM(1, 0, map(param.asInt(), 0, 180, SERVOMIN, SERVOMAX)); }

BLYNK_WRITE(V4) { pwm.setPWM(2, 0, map(param.asInt(), 0, 180, SERVOMIN, SERVOMAX)); }

BLYNK_WRITE(V5) { pwm.setPWM(3, 0, map(param.asInt(), 0, 180, SERVOMIN, SERVOMAX)); }

void checkDistance() {

digitalWrite(TRIG, LOW); delayMicroseconds(2);

digitalWrite(TRIG, HIGH); delayMicroseconds(10);

digitalWrite(TRIG, LOW);

long duration = pulseIn(ECHO, HIGH, 30000);

int distance = (duration == 0) ? 999 : (duration * 0.034 / 2);

Blynk.virtualWrite(V7, distance);

if (distance > 1 && distance <= 15 && !isRecoiling) {

isRecoiling = true;

drive(-255, -255);

timer.setTimeout(2000L, []() {

drive(0, 0);

isRecoiling = false;

});

}

}

BLYNK_WRITE(V0) { if(!isRecoiling) joyX = param.asInt(); }

BLYNK_WRITE(V1) { if(!isRecoiling) joyY = param.asInt(); }

void drive(int left, int right) {

left = constrain(left, -255, 255);

right = constrain(right, -255, 255);

if (left > 40) { digitalWrite(IN1, LOW); digitalWrite(IN2, HIGH); ledcWrite(ENA, abs(left)); }

else if (left < -40) { digitalWrite(IN1, HIGH); digitalWrite(IN2, LOW); ledcWrite(ENA, abs(left)); }

else { digitalWrite(IN1, LOW); digitalWrite(IN2, LOW); ledcWrite(ENA, 0); }

if (right > 40) { digitalWrite(IN3, LOW); digitalWrite(IN4, HIGH); ledcWrite(ENB, abs(right)); }

else if (right < -40) { digitalWrite(IN3, HIGH); digitalWrite(IN4, LOW); ledcWrite(ENB, abs(right)); }

else { digitalWrite(IN3, LOW); digitalWrite(IN4, LOW); ledcWrite(ENB, 0); }

}

void loop() {

Blynk.run();

timer.run();

if (!isRecoiling) drive(joyY + joyX, joyY - joyX);

}